Day 13 – 17.10.2025

Thejasvenan T.  220638J
ME 2261 - Embedded Systems Log Book

Task 1

Input character in virtual terminal while LED is blinking in PORT C.
The output for ASCII value of character will turn on at PORT B.

#define F_CPU 8000000UL

#include <avr/io.h>

#include <avr/interrupt.h>

#include <util/delay.h>

void USART_init(void) {

UCSRA = 0x00;

UCSRB = 0x98;

UCSRC = 0x86;

UBRRL = 0x33;

UBRRH = 0x00;

}

ISR(USART_RXC_vect) {

PORTB = UDR;

}

int main(void) {

USART_init();

sei();

DDRB = 0xFF;

DDRC = 0xFF;

while (1) {

PORTC |= (1 << PC0);

_delay_ms(2000);

PORTC &= ~(1 << PC0);

_delay_ms(2000);

}

return 0;

}

Task 2

There will be a delay in the same task 1 here when we interrupt the same task like this.

#define F_CPU 8000000UL

#include <avr/io.h>

#include <avr/interrupt.h>

#include <util/delay.h>

void USART_init(void) {

UCSRA = 0x00;

UCSRB = 0b00011000;

UCSRC = 0x86;

UBRRL = 0x33;

UBRRH = 0x00;

}

ISR(USART_RXC_vect) {

PORTB = UDR;

}

int main(void) {

USART_init();

sei();

DDRB = 0xFF;

DDRC = 0xff;

while (1) {

while((UCSRA & (1<<RXC)) == 0x00){

PORTC = 0x00000010;

_delay_ms(1000);

PORTC = 0x00000000;

_delay_ms(1000);

}

PORTB = UDR;

_delay_ms(1000);

}

return 0;

}

Task 3

When the push button is pressed or released, the PORTB becomes ~PORTB.

#define F_CPU 8000000UL

#include <avr/io.h>

#include <avr/interrupt.h>

#include <util/delay.h>

ISR (INT1_vect)

{

PORTB = (~PORTB);

}

int main(void)

{

GICR = (1<<INT1);

MCUCR = (1<<ISC10);

sei();

DDRB = 0xFF;

DDRC = 0X0;

PORTB = 0b01010101;

while (1){;}

return 0;

}

Task 4

When the push button is pressed & released, the PORTB becomes ~PORTB.

#define F_CPU 8000000UL

#include <avr/io.h>

#include <avr/interrupt.h>

#include <util/delay.h>

ISR (INT1_vect)

{

PORTB = (~PORTB);

}

int main(void)

{

GICR = (1<<INT1);

MCUCR = (1<<ISC10) | (1<<ISC11);

sei();

DDRB = 0xFF;

DDRC = 0X0;

PORTB = 0b01010101;

while (1){;}

return 0;

}

Task 5

Without prescaler

#define F_CPU 1000000UL

#include <avr/io.h>

#include <avr/interrupt.h>

#include <util/delay.h>

void timer0_init()

{

TCCR0 |= (1 <<CS00);

TCNT0 = 0;

}

int main(void){

DDRC |= (1 << 0);

timer0_init();

while(1){

if (TCNT0 >= 249)

{

PORTC ^= (1<<0);

TCNT0 = 0;

}

}

}

Task 6

Using prescaler

#define F_CPU 10000000UL

#include <avr/io.h>

#include <avr/interrupt.h>

#include <util/delay.h>

void timer0_init()

{

TCCR0 |= (1 <<CS02);	// prescaler 256

TCNT0 = 0;

}

int main(void){

DDRC |= (1 << 0);

timer0_init();

while(1){

if (TCNT0 >= 249)

{

PORTC ^= (1<<0);

TCNT0 = 0;

}

}

}